# foxBMS 2 — Software Architecture Specification

foxBMS 2 Lifecycle Artifact Corpus

2026-09-30

Table of Contents

# foxBMS 2 — Software Architecture Specification

Document Control

| Field | Value |
| --- | --- |
| Project | foxBMS 2 — Battery Management System |
| Document | Software Architecture Specification |
| Baseline | BAS-REF-001 (commit 308028fb, tag v1.11.0) |
| Profiles | as_is (source-grounded) + synthetic_reference (hypothetical) |
| Corpus status | synthetic_ready_with_limitations |
| Generated | 2026-09-30T01:21:29Z |

## Scope

Software architecture viewpoints: reverse-engineered static component viewpoint (40-module inventory across 4 layers with prefixes and unit tests), dynamic viewpoints (BMS/SYS state machines, task model), plus the corpus SWR/DSN relations per profile.

## Static Component Viewpoint — Module Inventory

All software modules reverse-engineered from src/app/ (layer / module / prefix / brief / sources / unit tests).

### Layer: engine (5 modules)

| Module | Prefix | Responsibility (from @brief) | Files | Unit tests |
| --- | --- | --- | --- | --- |
| engine/database | DATA | Database module header | 4 | 2 |
| engine/diag | DIAG | Diagnosis driver header | 24 | 1 |
| engine/hw_info | MINFO | General foxBMS-master system information | 2 | 1 |
| engine/sys | SYS | Software reset driver header | 4 | 2 |
| engine/sys_mon | SYSM | System monitoring module | 2 | 1 |

### Layer: task (3 modules)

| Module | Prefix | Responsibility (from @brief) | Files | Unit tests |
| --- | --- | --- | --- | --- |
| task/ftask | FTSK | Header of task driver implementation | 3 | 2 |
| task/os | OS | Declaration of the OS wrapper interface | 4 | 1 |
| task/timer | TIMER | Header file for the timer wrapper | 2 | 1 |

### Layer: application (7 modules)

| Module | Prefix | Responsibility (from @brief) | Files | Unit tests |
| --- | --- | --- | --- | --- |
| application/algorithm | ALGO | Algorithms framework: SOX estimation and moving averages | 26 | 1 |
| application/bal | BAL | Header for the driver for balancing | 5 | 1 |
| application/bms | BMS | BMS driver header | 2 | 1 |
| application/ethernet | ETH | Header of the ethernet software | 4 | 2 |
| application/plausibility | PL | Plausibility checks for cell voltage and cell temperatures | 2 | 1 |
| application/redundancy | MRC | Header files for handling redundancy between redundant cell voltage | 2 | 1 |
| application/soa | SOA | Header for SOA module, responsible for checking battery parameters | 2 | 1 |

### Layer: driver (25 modules)

| Module | Prefix | Responsibility (from @brief) | Files | Unit tests |
| --- | --- | --- | --- | --- |
| driver/adc | ADC | Headers for the driver for the ADC module. | 2 | 1 |
| driver/can | CAN | Header for the driver for the CAN module | 44 | 4 |
| driver/contactor | CONT | Headers for the driver for the contactors. | 2 | 1 |
| driver/crc | CRC | CRC module header | 2 | 1 |
| driver/dma | DMA | Headers for the driver for the DMA module. | 2 | 4 |
| driver/emac | EMAC | Implementation of emac driver | 4 | 2 |
| driver/foxmath | MATH | Math library for often used math functions | 4 | 2 |
| driver/fram | FRAM | Header for the driver for the FRAM module | 2 | 1 |
| driver/htsensor | HTSEN | Header for the driver for the Sensirion SHT35-DIS I2C humidity/temperature sensor | 2 | 1 |
| driver/i2c | I2C | Header for the driver for the I2C module | 2 | 1 |
| driver/imd | IMD | API header for the insulation monitoring device | 12 | 1 |
| driver/interlock | ILCK | Headers for the driver for the interlock. | 2 | 1 |
| driver/io | IO | Header for the driver for the IO module | 2 | 1 |
| driver/led | LED | Header file of the debug LED driver | 2 | 1 |
| driver/mcu | MCU | Headers for the driver for the MCU module. | 2 | 1 |
| driver/meas | MEAS | Headers for the driver for the measurements needed by the BMS | 2 | 1 |
| driver/pex | PEX | Header for the driver for the NXP PCA9539 port expander module | 2 | 1 |
| driver/phy | PHY | Implementation of physical layer driver | 2 | 1 |
| driver/pwm | PWM | PWM driver for the TMS570LC43xx. | 2 | 1 |
| driver/rtc | RTC | Header file of the RTC driver | 2 | 1 |
| driver/sbc | FS85 | Driver for the NXP FS85 SBC (system basis chip) supervisor | 11 | 3 |
| driver/spi | SPI | Headers for the driver for the SPI module. | 3 | 9 |
| driver/sps | SPS | Headers for the driver for the smart power switches. | 3 | 1 |
| driver/ts | BETA | Temperature sensor evaluation (resistive divider, beta model) | 44 | 1 |
| driver/uart | UART | Drivers for UART RS232 | 2 | 2 |

Configuration is separated from module logic into per-layer config/ directories (src/app/*/config/*_cfg.c|h) — each module pairs with a *_cfg file (see Detailed Design).

## Dynamic Viewpoint — Task Model

Caption: RTOS task set (FreeRTOS): four cyclic tasks (1 ms, 10 ms, 100 ms, 100 ms algorithm) plus continuous blocking tasks (I2C, engine). Source: src/app/task/ftask/ftask.c, src/app/task/config/ftask_cfg.c.

flowchart TD
    OS[FreeRTOS Scheduler]
    1ms["1ms (4 functions)"]
    OS --> 1ms
    10ms["10ms (11 functions)"]
    OS --> 10ms
    100ms["100ms (5 functions)"]
    OS --> 100ms
    100ms_algorithm["100ms-algorithm (1 functions)"]
    OS --> 100ms_algorithm
    i2c_continuous["i2c (continuous) (3 functions)"]
    OS --> i2c_continuous
    engine_continuous["engine (continuous) (1 functions)"]
    OS --> engine_continuous

## Data Exchange Viewpoint

Caption: Producer/consumer database (engine layer) — asynchronous data exchange between tasks; single producer, multiple consumers, integrity ensured (source: docs/software/structure/application.rst, src/app/engine/database/).

flowchart LR
    MEAS[MEAS_Control\n1ms] --> DB[(Database DATA)]
    CANRX[CAN_ReadRxBuffer\n1ms] --> DB
    ADC[ADC_Control\n10ms] --> DB
    SPS[SPS_Ctrl\n10ms] --> DB
    MRC[MRC_Validate*\n50ms] --> DB
    DB --> SOA[SOA evaluation]
    DB --> ALGO[SE_RunStateEstimations\n1s]
    DB --> BMS[BMS_Trigger\n10ms]
    BMS --> CAN_TX[CAN_MainFunction\n10ms]
    DIAG[DIAG_UpdateFlags\n1ms] --> DB

## Profile: as_is — Traceability Viewpoints

### Static Component Viewpoint (corpus)

Caption: SW requirements, designs, and implements/allocated_to relations.

flowchart TD
    "FB2-SW-SWR-000001" --> "FB2-SAF-FSR-000001"
    "FB2-SW-SWR-000002" --> "FB2-SAF-FSR-000002"
    "FB2-SW-SWR-000003" --> "FB2-SAF-FSR-000003"
    "FB2-SW-DSN-000001" -.implements.-> "FB2-SW-SWR-000001"
    "FB2-SW-DSN-000002" -.implements.-> "FB2-SW-SWR-000002"
    "FB2-SW-DSN-000003" -.implements.-> "FB2-SW-SWR-000003"

### Dynamic Viewpoint — FB2-SW-DSN-000001

Caption: State machine of FB2-SW-DSN-000001 from behavior_model (state_machine).

stateDiagram-v2
    [*] --> INIT
    INIT --> WAKEUP : driver_init
    WAKEUP --> READ_VOLTAGES : wakeup_done
    READ_VOLTAGES --> READ_TEMPERATURES : voltages_done
    READ_TEMPERATURES --> BALANCING : temps_done
    BALANCING --> READ_VOLTAGES : cycle_timer
    * --> ERROR : pec_failure
    ERROR --> INIT : reinit_request
    INIT
    WAKEUP
    READ_VOLTAGES
    READ_TEMPERATURES
    BALANCING
    ERROR
    SLEEP

### Dynamic Viewpoint — FB2-SW-DSN-000002

Caption: State machine of FB2-SW-DSN-000002 from behavior_model (state_machine).

stateDiagram-v2
    [*] --> IDLE
    IDLE --> CHECKING : database_updated
    CHECKING --> DEBOUNCING : limit_exceeded
    DEBOUNCING --> VIOLATION_CONFIRMED : debounce_complete
    DEBOUNCING --> IDLE : back_in_limits
    VIOLATION_CONFIRMED --> FAULT_TRIGGERED : fault_callback
    FAULT_TRIGGERED --> IDLE : system_reset
    IDLE
    CHECKING
    DEBOUNCING
    VIOLATION_CONFIRMED
    FAULT_TRIGGERED

### Dynamic Viewpoint — FB2-SW-DSN-000003

Caption: State machine of FB2-SW-DSN-000003 from behavior_model (state_machine).

stateDiagram-v2
    [*] --> OPEN
    OPEN --> PRECHARGE : request_precharge
    PRECHARGE --> PRECHARGE_WAIT : precharge_contactor_closed
    PRECHARGE_WAIT --> CLOSE : voltage_threshold_reached
    CLOSE --> HOLD : main_contactor_closed
    HOLD --> FAULT_OPEN : fault_request
    HOLD --> OPEN : request_open
    * --> FAULT_OPEN : fault_request
    HOLD --> WELD_DETECTED : feedback_mismatch
    HOLD --> STUCK_OPEN : feedback_mismatch
    OPEN
    PRECHARGE
    PRECHARGE_WAIT
    CLOSE
    HOLD
    FAULT_OPEN
    WELD_DETECTED
    STUCK_OPEN

## Profile: synthetic_reference — Traceability Viewpoints

### Static Component Viewpoint (corpus)

Caption: SW requirements, designs, and implements/allocated_to relations.

flowchart TD
    "FB2-SW-SWR-000001" --> "FB2-SAF-FSR-000001"
    "FB2-SW-SWR-000001" --> "FB2-SYS-SYR-000001"
    "FB2-SW-SWR-000001" --> "FB2-SYS-SYR-000002"
    "FB2-SW-SWR-000001" --> "FB2-SYS-SYR-000006"
    "FB2-SW-SWR-000001" --> "FB2-SYS-SYR-000007"
    "FB2-SW-SWR-000002" --> "FB2-SAF-FSR-000002"
    "FB2-SW-SWR-000002" --> "FB2-SYS-SYR-000002"
    "FB2-SW-SWR-000003" --> "FB2-SAF-FSR-000003"
    "FB2-SW-SWR-000003" --> "FB2-SYS-SYR-000003"
    "FB2-SW-SWR-000003" --> "FB2-SYS-SYR-000004"
    "FB2-SW-SWR-000003" --> "FB2-SYS-SYR-000008"
    "FB2-SW-DSN-000001" -.implements.-> "FB2-SW-SWR-000001"
    "FB2-SW-DSN-000002" -.implements.-> "FB2-SW-SWR-000002"
    "FB2-SW-DSN-000003" -.implements.-> "FB2-SW-SWR-000003"

### Dynamic Viewpoint — FB2-SW-DSN-000001

Caption: State machine of FB2-SW-DSN-000001 from behavior_model (state_machine).

stateDiagram-v2
    [*] --> INIT
    INIT --> WAKEUP : driver_init
    WAKEUP --> READ_VOLTAGES : wakeup_done
    READ_VOLTAGES --> READ_TEMPERATURES : voltages_done
    READ_TEMPERATURES --> BALANCING : temps_done
    BALANCING --> READ_VOLTAGES : cycle_timer
    * --> ERROR : pec_failure
    ERROR --> INIT : reinit_request
    INIT
    WAKEUP
    READ_VOLTAGES
    READ_TEMPERATURES
    BALANCING
    ERROR
    SLEEP

### Dynamic Viewpoint — FB2-SW-DSN-000002

Caption: State machine of FB2-SW-DSN-000002 from behavior_model (state_machine).

stateDiagram-v2
    [*] --> IDLE
    IDLE --> CHECKING : database_updated
    CHECKING --> DEBOUNCING : limit_exceeded
    DEBOUNCING --> VIOLATION_CONFIRMED : debounce_complete
    DEBOUNCING --> IDLE : back_in_limits
    VIOLATION_CONFIRMED --> FAULT_TRIGGERED : fault_callback
    FAULT_TRIGGERED --> IDLE : system_reset
    IDLE
    CHECKING
    DEBOUNCING
    VIOLATION_CONFIRMED
    FAULT_TRIGGERED

### Dynamic Viewpoint — FB2-SW-DSN-000003

Caption: State machine of FB2-SW-DSN-000003 from behavior_model (state_machine).

stateDiagram-v2
    [*] --> OPEN
    OPEN --> PRECHARGE : request_precharge
    PRECHARGE --> PRECHARGE_WAIT : precharge_contactor_closed
    PRECHARGE_WAIT --> CLOSE : voltage_threshold_reached
    CLOSE --> HOLD : main_contactor_closed
    HOLD --> FAULT_OPEN : fault_request
    HOLD --> OPEN : request_open
    * --> FAULT_OPEN : fault_request
    HOLD --> WELD_DETECTED : feedback_mismatch
    HOLD --> STUCK_OPEN : feedback_mismatch
    OPEN
    PRECHARGE
    PRECHARGE_WAIT
    CLOSE
    HOLD
    FAULT_OPEN
    WELD_DETECTED
    STUCK_OPEN

### Task/Thread Timing Context

Caption: Timing budget elements (from FB2-SAF-SGO-000001) as scheduling context.

flowchart LR
    CHAIN["Protection chain"]
    T_afe_acquisition_ms["afe_acquisition_ms<br/>25 ms"]
    T_afe_acquisition_ms --> CHAIN
    T_contactor_command_ms["contactor_command_ms<br/>5 ms"]
    T_contactor_command_ms --> CHAIN
    T_contactor_mechanical_ms["contactor_mechanical_ms<br/>30 ms"]
    T_contactor_mechanical_ms --> CHAIN
    T_database_publish_ms["database_publish_ms<br/>3 ms"]
    T_database_publish_ms --> CHAIN
    T_fault_classification_ms["fault_classification_ms<br/>2 ms"]
    T_fault_classification_ms --> CHAIN
    T_feedback_verification_ms["feedback_verification_ms<br/>5 ms"]
    T_feedback_verification_ms --> CHAIN
    T_margin_ms["margin_ms<br/>15 ms"]
    T_margin_ms --> CHAIN
    T_pec_validation_ms["pec_validation_ms<br/>2 ms"]
    T_pec_validation_ms --> CHAIN
    T_soa_limit_check_ms["soa_limit_check_ms<br/>5 ms"]
    T_soa_limit_check_ms --> CHAIN
    T_spi_transfer_ms["spi_transfer_ms<br/>5 ms"]
    T_spi_transfer_ms --> CHAIN
    T_sys_state_transition_ms["sys_state_transition_ms<br/>3 ms"]
    T_sys_state_transition_ms --> CHAIN

Generated: 2026-09-30T01:21:29Z — auto-generated from the machine-verifiable corpus. Regenerate with python3 docs/artifacts/tools/render_spec_documents.py.